# UM9800 V.90 USB MODEM AT COMMAND SET REFERENCE

1 - AT Command Set

Modem operation is controlled by AT commands. These AT commands include the following:

Basic AT commands, for example ATDT123

Extended AT commands, for example AT&E, AT\A, AT%C, AT+MS

S-Register commands, for example ATS0=1

Fax Class 1 commands, for example AT+FTM

Voice commands, for example AT#VTX

The command syntax and operation guidelines for each command category are described in the following sections.

A command line is a string of characters sent from a DTE (Terminal or Data Terminal Equipment) to the DCE (Data Terminal Circuit Equipment) while the DCE is in a command state. Command lines have a prefix, a body and a terminator. The prefix consists of the ASCII character AT or at. The body consists of printable ASCII characters. Space characters other than <CR> (see register S3) and <BS> (see register S5) are ignored. <CR> is command terminator.

Characters preceding the AT prefix are ignored.

AT Command Guidelines:

Basic AT commands consist of single ASCII characters which may be preceded by a prefix character, for example &, and followed by a decimal number, for example AT&W1.

Missing decimal parameters are interpreted as 0. For example, if ATH is typed, the command ATH0 is assumed.

Fax commands are preceded with the +F characters and terminated by semicolon (;) or <CR> character.

The modem supports editing command lines by recognizing the <BS> character.

The AT command sequence may be followed by any number of commands sequence with the exception of commands Z, D or A where all characters following on the same command line will be ignored.

When a syntax error is found in the command line an ERROR response will be returned to the DTE. Execution of commands D and A will be aborted if another character is entered before completion of the handshake.

When the modem has entered on-line data mode, it is possible to break the data transmission in order to issue more AT commands. This is done by the DTE sending a sequence of three escape characters defined in S2, ‘+’ by default.

The modem will comply with the commands listed below. Parameters applicable to each command are listed below. Default factory settings are mark by an asterisk *.

Features marked with (-) are not yet available in current version of Modem.

### 1.1 - Basic AT Commands

A/		Re-execute Command

The modem repeats the last command line sent by the DTE. Usually used for re-dialing.

Note:	This command should not be terminated by <CR>.

A	Answer

The modem will go off-hook and attempt to answer an incoming call. Upon successful completion of handshake, the modem will go on-line in answer mode.

Notes:

If +FCLASS=0 is selected, the modem will enter the connected state after exchanging carrier with the remote system. If no carrier is detected within the period specified in S7, the modem hangs up. Any character entered while connecting will abort the connection process.

If +FCLASS=1, the modem will go off-hook in V21 answer mode. It will generate the V21 2100 Hz answer tone for 3 +/- 0.5 seconds, and following a delay of 70 ms, will proceed as if the +FTH=3 command were issued. At any stage up to (but excluding) the +FTH=3 command state, any character will abort the communication.

If +FCLASS=8 (#CLS=8), the modem will go off-hook and a voice session will take place.

Related S-Reg: S0

Bn	CCITT Control

B0	Connect at V.22 1200 bps

Result codes:

OK	n=0

Error 	Otherwise

Dn	Dial

Directs the modem to go on-line, dial according to the string entered, and attempt to establish a connection.

The Dial String may consist of any of the characters described below:

T	Tone dialing (first character in the string)

P	Pulse dialing (first character in the string)

L	Redial last dialed number (first character in the string)

0-9	Digits 0 to 9

- *	Asterisk (tone only)

- #	Hash (tone only)

W	Wait for dial tone; the modem will wait for dial tone before dialing the digits following “W”. S6 register will be used for timeout. (X3 or higher)

,	(Comma); Pause for the time specified by S8 before resuming the dialing

;	(Semicolon) Return to command mode after dialing. This allows the user to issue additional AT commands while remaining off-hook. Actual call progress will be entered only after a dial command issued without the “;” terminator.

S=n	Dial the number stored in the directory; n=0-3 (see &Z).

!	(NA in V1.0x) Flash. The modem will go on hook for a time defined by S24.

@	(NA in V1.0x) Wait for silence. The modem will wait for at least 5 seconds of silence before resuming the dialing. If no such silence is detected before the expiration of the call abort timer (S7), the modem will terminate with NO ANSWER response (or BUSY if applicable). If answer tone arrives during the execution of this parameter, the modem will do handshaking. (X3 or higher)

(),< >	(space) String format characters - ignored

<i>	any other character - ignored.

Notes:

If +FCLASS=0 is selected, the modem will attempt to connect with another data modem. The modem will use the time period specified in S6 and S7 as time-outs in the handshake process. If a time out expires, the modem will go on-hook and respond with NO CARRIER response. The command will be aborted in progress is a DTE character is entered before completion of the handshake.

If +FCLASS=1, the modem will attempt to connect with a fax machine (or modem) by entering the HDLC V21 channel 2 receive state (as if +FRH=3 had been issued).

The command will be aborted upon receipt of a DTE character if the modem has not finished dialing. In this case the modem will go on-hook and return to command mode responding with NO CARRIER message. If the modem has finished dialing, It proceeds as if +FRH=3 command has been issued.

If +FCLASS=8 (#CLS=8), the modem will go off-hook in V21 answer mode. It will decide (based on timers) when the other side answers in voice and a voice session will take place.

Related S-Reg:  S5, S6, S7, S16, S22, S28, S56

En	Set local echo

The modem enables/disables echo of characters to DTE.

Parameter value is written to S13.

E0	Disable command echo.

- E1	Enable command echo (Default)

Result codes:

OK	n=0 or 1

Error	Otherwise

Related S-Reg: S13

Hn	Set ON/OFF hook

H0	Modem hangs up (goes on-hook).

H1	Modem goes off hook.

Result codes:

OK 	n=0 or 1

Error 	Otherwise

In	Identification/Information

I1	UM9800 V.90 USB Modem

AZT

Ver2.40

I2	AZT

Ver2.40

MV (CID/SPK)

I3	AZT

ST7554

I4	Active Profile:

S00=000 S01=000 S02=043 S03=013 S04=010 S05=008 S06=003 S07=035

S08=002 S09=006 S10=128 S11=105 S12=050 S13=001 S14=000 S15=001

S16=001 S17=000 S18=000 S19=000 S20=000 S21=000 S22=000 S23=000

S24=020 S25=000 S26=000 S27=008 S28=000 S29=001 S30=002 S31=001

S32=090 S33=009 S34=034 S35=000 S36=000 S37=000 S38=000 S39=002

S40=000 S41=002 S42=000 S43=000 S44=000 S45=251 S46=000 S47=000

S48=000 S49=000 S50=000 S51=000 S52=000 S53=000 S54=000 S55=000

S56=004 S57=000 S58=000 S59=000 S60=001 S61=000 S62=001 S63=000

S64=000 S65=000 S66=000 S67=000 S68=000 S69=000 S70=000 S71=000

S72=019 S73=003 S74=153 S75=000 S76=004 S77=010 S78=000 S79=070

S80=070 S81=000 S82=002 S83=055 S84=000 S85=000 S86=000 S87=000

S88=001

I5	Stored profile 0

Not Defined

I6	Stored profile 1

Not Defines

I7	Country: FRANCE

Serial Number: 0

Protocol Factor: 0

Ln	Speaker volume

Select speaker volume.

L0	Low

L1	Low (Default)

L2	Medium

L3	High

Result codes:

OK	n=0-3

Error	Otherwise

Related S-Reg: S30

Mn	Speaker control

Select when the speaker is On/Off.

M0	Speaker always OFF

- M1	Speaker ON from start of dialing until receiving carrier (Default)

M2	Speaker always ON

M3	Speaker OFF from end of dialing until receiving carrier

Result codes:

OK	n=0-3

Error	Otherwise

Related S-Reg: S29

Nn	Automode control

Enable/Disable Automode detection.

N0	Automode detection disabled. A subsequent handshake will be conducted according to the contents of S32.

N1	Automode enabled. A subsequent handshake will be conducted according to the Automode algorithm (Default)

Result codes:

OK	n=0 or 1

Error	Otherwise

Related S-Reg: S31

On	Returns to on-line data mode

This command is normally used to connect the DTE back after an escape (+++) has been issued.

O0	Return to on-line data mode.

O1	Return to on-line data mode, retrain first.

Result codes:

OK	n=0-1

Error	Otherwise

P	Pulse dialing

Forces Pulse dialing. Applies to subsequent dialing commands.

This command holds until the next T dial modifier or T command is received.

The modem will go off hook and attempt to answer an incoming call. Upon successful completion of handshake, the modem will go on-line in answer mode.

Related S-Reg: S16

Q	Quiet result codes control

- Q0	Enable sending result codes to DTE (Default).

Q1	Disable sending result codes to DTE.

Result codes:

OK	n=0 or 1

Error	Otherwise

Related S-Reg: S14

S	Read/Write S-Register

This command has a few derivatives:

Sn=v	Sets the value v (decimal) to S-register n (v=0-255)

Sn?	Displays the value of S-register in decimal format (3 digits)

Note:	Some registers are read-only

Result codes:

OK	All parameters valid

Error 	Invalid S register or value. Trying to write to a read-only register

T	Tone dialing

Forces tone dialing. Applies to subsequent dialing commands. This command holds until the next T dial modifier or T command is received.

This command changes S14 to reflect the current dialing mode.

Related S-Reg: S16

Vn	Verbose/Numeric result codes

Select the time of result messages sent to the DTE.

For a list of result codes and verbal messages see X command.

V0	Short form (numeric) result codes to be sent to DTE.

- V1	Long form (verbose) result codes to be sent to DTE (Default).

Result codes:

OK	n=0 or 1

Error	otherwise

Related S-Reg: S15

Xn	Extended result codes

Select the subset of result codes to be used by the modem to the DTE.

If the modem is in fax mode (+FCLASS=1), the only message sent to indicate connection is “CONNECT” without a speed indication.

X0	Supported messages: OK, CONNECT, RING, NO CARRIER and ERROR, Blind call enabled.

X1	Supported messages: OK, CONNECT xxxx, RING, NO CARRIER and ERROR, Blind call enabled.

X2	Same as X1 + NO DIAL TONE message, Blind call disabled

X3	Same as X1 + BUSY message, Blind call enabled.

- X4	All messages supported, Blind call disabled (Default).

Notes:

W,@ dial modifiers are ignored in X1, X2

S6 (Wait before dial) is ignored in X2, X4 if no W is specified in dial string

S6 is set to 0 means a blind call

#### Table 1: Result Codes and Messages

| Result Code | Message | X0 | x1 | x2 | x3 | x4 |
| --- | --- | --- | --- | --- | --- | --- |
| 0.00 | OK | * | * | * | * | * |
| 1.00 | CONNECT | * | * | * | * | * |
| 2.00 | RING | * | * | * | * | * |
| 3.00 | NO CARRIER | * | * | * | * | * |
| 4.00 | ERROR | * | * | * | * | * |
| 5.00 | CONNECT 1200 | 1.00 | * | * | * | * |
| 6.00 | NO DIAL TONE | 3.00 | 3.00 | * | 3.00 | * |
| 7.00 | BUSY | 3.00 | 3.00 | 3.00 | * | * |
| 8.00 | NO ANSWER | 3.00 | 3.00 | 3.00 | * | * |
| 9.00 | CONNECT 0300 | 1.00 | * | * | * | * |
| 10.00 | CONNECT 0600 | 1.00 | * | * | * | * |
| 11.00 | CONNECT 2400 | 1.00 | * | * | * | * |
| 12.00 | CONNECT 4800 | 1.00 | * | * | * | * |
| 13.00 | CONNECT 7200 | 1.00 | * | * | * | * |
| 27.00 | CONNECT 9600 | 1.00 | * | * | * | * |
| 14.00 | CONNECT 12000 | 1.00 | * | * | * | * |
| 15.00 | CONNECT 14400 | 1.00 | * | * | * | * |
| 16.00 | CONNECT 16800 | 1.00 | * | * |  |  |
| 17.00 | CONNECT 19200 | 1.00 | * | * | * | * |
| 18.00 | CONNECT 21600 | 1.00 | * |  |  |  |
| 19.00 | CONNECT 24000 | 1.00 | * | * | * | * |
| 20.00 | CONNECT 26400 | 1.00 | * | * | * | * |
| 21.00 | CONNECT 28800 | 1.00 | * | * | * | * |
| 22.00 | CONNECT 31200 | 1.00 | * | * | * | * |
| 40.00 | CONNECT 32000 | 1.00 | * | * | * | * |
| 23.00 | CONNECT 33600 | 1.00 | * | * | * | * |
| 24.00 | CONNECT 34800 | 1.00 | * | * | * | * |
| 25.00 | CONNECT 40000 | 1.00 | * | * | * | * |
| 26.00 | CONNECT 42000 | 1.00 | * | * | * | * |
| 28.00 | CONNECT 44000 | 1.00 | * | * | * | * |
| 29.00 | CONNECT 46000 | 1.00 | * | * | * | * |
| 30.00 | CONNECT 48000 | 1.00 | * | * | * | * |
| 31.00 | CONNECT 50000 | 1.00 | * | * | * | * |
| 32.00 | CONNECT 52000 | 1.00 | * | * | * | * |
| 33.00 | CONNECT 54000 | 1.00 | * | * | * | * |
| 34.00 | CONNECT 56000 | 1.00 | * | * | * | * |
| 66.00 | COMPRESSION: CLASS 5 | - | * | * | * | * |
| 67.00 | COMPRESSION: V.42BIS | - | * | * | * | * |
| 69.00 | COMPRESSION: NONE | - | * | * | * | * |
| 70.00 | FAX | * | * | * | * | * |
| 71.00 | DATA | * | * | * | * | * |
| 76.00 | PROTOCOL: NONE | - | * | * | * | * |
| 77.00 | PROTOCOL: LAPM | - | * | * | * | * |
| 78.00 | PROTOCOL: MNP | - | * | * | * | * |
| 1021.00 | MODULATION: V.21 | - | * | * | * | * |
| 1022.00 | MODULATION: V.22 | - | * | * | * | * |
| 1032.00 | MODULATION: V.32 | * | * | * | * |  |
| 1034.00 | MODULATION: V.34 | - | * | * | * | * |
| 1103.00 | MODULATION: B103 | - | * | * | * | * |
| 1122.00 | MODULATION: V.22BIS | - | * | * | * | * |
| 1132.00 | MODULATION: V.32BIS | - | * | * | * | * |
| 1134.00 | MODULATION: V.34BIS | - | * | * | * | * |
| 1212.00 | MODULATION: B212 | - | * | * | * | * |
| +F4 | +FCERROR | * | * | * | * | * |

<*> message will be generated when n has been selected

<i> message will be replaced by message <i> when n has been selected

<-> message will not be generated when n has been selected.

Related S-Reg: S56

Yn	Select default configuration

Select the default user defined configuration.

Note:	The default configuration is not loaded by Yn (See Zn)

Y0	Select user template 0

Y1	Select user template 1

- Y2	Select factory setting 0 (Default)

Y3	Select factory setting 1

Related S-Reg: S161

Zn	Select user defined configuration

Select the user defined configuration.

Z0	Select default user template (as defined by Yn)

Z1	Select user template 0

Z2	Select user template 1

Z3	Select factory setting 0 (&F0)

Z4	Select factory setting 1 (&F1)

Result codes:

OK	n=0-5

Error	Otherwise

Related S-Reg: S59

### 1.2 - AT& Commands

&An	Connect message format

Select the format of the CONNECT message.

- &A0	no extra messages besides CONNECT xxxxx (Default)

&A1	Add Modulation indicator:

V.21/ V.22/ V.22BIS/ V.32/ V.32BIS/ V.34/ V.34BIS/ B103/ B212

For example:

Modulation: V.34

&A2	Add Error Detection Protocol and Data Compression indicators.

For example:

Protocol: LAPM/MNP/NONE

Compression: CLASS 5/V.42BIS/NONE

&A3	Add Modulation Indicator + Error Detection Protocol + Data Compression indicators (see above).

Related S-Reg: S70, S71

&Cn	Control Carrier Detect (CD, RLSD) behavior

Controls the RLSD output behavior.

&C0	RLSD is assumed to be ON all the time

- &C1	RLSD follows the carrier state (Default)

Result codes:

OK	n=0 or 1

Error	Otherwise

Related S-Reg: S60

&Dn	Controls DTR behavior (NA)

Controls the DTR output behavior.

Note:	This command is supported for compatibility. It has no significance in Modem environment.

&D0	DTR is taken to be ON all the time

- &D1	DTR drop causes entry to command mode without disconnect (Default)

&D2	DTR follows DTR circuit definition

&D3	DTR drop causes software reset (as in Z0)

Result codes:

OK	n=0-3

Error	Otherwise

Related S-Reg: S63

&En	Connect message speed source

Select the requested source for the speed field in the CONNECT message.

&E0	DCE Speed

&E1	DTE Speed

Note:	Since a virtual port is involved, the DTE is not bound by any UART limitation and may be theoretically set as high as 921600. DTE speed is supported for compatibility only. It bears little significance in Modem environment.

Related S-Reg: S71

&Fn	Sets factory configuration

Select one of the factory settings.

&F0	Select factory setting 0

&F1	Select factory setting 1

Result codes:

OK	n=0-1

Error	Otherwise

Related S-Reg: S59

&Hn	Sets flow control

Select the user defined configuration.

&H0	Flow control disabled (NA)

- &H1	“HW” flow control RTS/CTS emulation (Default)

Result codes:

OK	n=0-1

Error	Otherwise

Related S-Reg: S62

&Pn	Set pulse dial make/break ratio

- &P0	US & Canada 39%/61% (10 pps) (Default)

&P1	UK & Hong Kong 33%/67% (10 pps)

&P2	Same as 0, except at 20 pps

&P3	Same as 1, except at 20 pps

Result codes:

OK	n=0-3

Error	Otherwise

Related S-Reg: S28

&Rn	Controls RTS behavior

Controls the RTS output behavior.

Note:	This command is supported for compatibility. It has no significance in Modem environment

&R0	RTS ignored

&R1	Modem receives data only on RTS (NA)

Result codes:

OK	n=0 or 1

Error	Otherwise

Related S-Reg: S61

&Sn	Controls DSR behavior

Note:	This command is supported for compatibility. It has no significance in Modem environment.

&S0	DSR override (is assumed to be ON all the time)

&S1	DSR follows circuit definition

Result codes:

OK	n=0 or 1

Error	Otherwise

Related S-Reg: S64

&Tn	Test mode

- &T0	TBD (Default)

&T1	TBD

&T2	TBD

&T3	TBD

Result codes:

OK	n=0-3

Error	Otherwise

Related S-Reg: S18, S50-S55

&V	Display Verbose Information

Display Active profile, Stored Profiles, Stored Phone Numbers

(Equivalent to I4-I7 combined)

&Wn	Writes current configuration

&W0	Write to template 0

&W1	Write to template 1

Result codes:

OK	n=0-1

Error	Otherwise

Written to registry.

&Zn	Stores dial string

Stores/Displays dial string (up to 47 characters)

&Zn=s	Store dial string (n=0-4)

&Zn=L	Store the last dialed string (n=0-4)

&Zn?	Display the nth string

&ZL?	Display the last dialed string

Written to registry.

### 1.3 - AT \ Commands - Error correction control

\An	Maximum MNP block Size

\A0	64 characters maximum block size

\A1	128 characters maximum block size

\A2	192 characters maximum block size

\A3	256 characters maximum block size

Result codes:

OK	n=0-3

Error	Otherwise

Related S-Reg: S<basereg+1> of V.42 registers

\Bn	Transmit break to remote (-)

In non-error correction mode, the modem will transmit a break signal to the remote modem with a length of n*100ms. If a number above 9 is entered, 9 is used.

Result codes:

OK	if connected in data modem mode

Error	if not connected or if connected in fax modem mode

\Kn	Break Control (-)

Controls the response of the modem to a break received from DTE or a remote modem or the \Bn command.

#### The behavior parameter is written to Sxx

\K0	Enter on-line command mode, no break sent to remote modem

\K1	Clear data buffers and send break to remote modem

\K2	Same as 0

\K3	Send break to remote modem immediately

\K4	Same as 0

\K5	Send a break to remote modem in sequence with transmitted data (Default)

Related S-Reg: S<basereg+x> of V.42 registers

Result codes:

OK	n=0-5

Error	Otherwise

\Nn	Error correction operating mode

\N0	Normal (Speed buffering) - No error correction

\N1	Direct (pass-through) 128 characters maximum block size

\N2	Reliable (error correction) mode. The Modem will attempt LAPM and then MNP

\N3	Auto reliable mode. Same as \N2, but will fall back to Normal

\N4	LAPM error correction mode only, hang up upon failure (Default)

\N5	MNP error correction mode only, hang up upon failure

Result codes:

OK	n=0-5

Error	Otherwise

Related S-Reg: S<basereg> of V.42 registers

### 1.4 - AT% Commands

%Cn	Compression control

%C0	Disable data compression

%C1	Enable MNP5 data compression

%C2	Enable V.42bis data compression

- %C3	Enable MNP5/V.42bis data compression (Default)

Result codes:

OK	n=0-3

Error	Otherwise

Related S-Reg: S<basereg+2> of V.42 registers

%En	Line quality monitor control

Controls whether or not the modem will automatically monitor the line quality and request a retrain (%E1), or fall back when quality is insufficient or fall forward when line quality improves (%E2).

#### %E0	Disable line quality control

#### %E1	Enable line quality control and auto retrain

- %E2	Enable line quality control and fallback/forward

- Result codes:

OK		n=0-3

Error	Otherwise

Related S-Reg: S39

1.5 - AT+MS Command

+MS	Modulation select

This command selects the modulation, optionally enables/disables Automode, and optionally specifies the lowest and highest connection rates.

The command format is:

AT+MS= [<mod>][,[<automode>][,[<min_rate>][,[<max_rate>]]]]

<mod>	a decimal number specifying the preferred modulation (automode enabled), or the modulation (automode disabled).

<automode>	0/1 Automode disabled/enabled

<min_rate>	minimum rate for connection. If lower than the actual minimum rate for the selected modulation, the actual lowest supported rate will be taken.

<max_rate>	maximum rate for connection. If higher than the actual maximum rate for the selected modulation, the actual highest supported rate will be taken.

Table 2: +MS command parameters

| <mod> | Modulation | Possible rates |
| --- | --- | --- |
| 22.00 | V.22 | 1200.00 |
| 122.00 | V.22bis | 2400, 1200 |
| 32.00 | V.32 | 9600, 4800 |
| 132.00 | V.32bis | 14400, 12000, 9600, 7200, 4800 |
| 34.00 | V.34 | 33600, 31200, 28800, 26400, 24000, 21600, 19200,16800, 14400, 12000, 9600 |
| 212.00 | Bell 212 | 1200.00 |
| 103.00 | Bell 103 | 300.00 |

Examples:

AT+MS=34,0,4800,33600 	V.34, No Automode, Min. speed 4800, Max speed 33600

AT+MS=,1 			Automode

AT+MS=32,1,,14400 		V.32 Automode, Max speed 14400 (min speed as before)

Factory Settings: 34,1,300,33600

The requested modulation scheme will be written to S32

The requested min rate will be written to S33

The requested max rate will be written to S34

The actual rate may be read from S35

The actual modulation scheme may be read from S37

(codes as specified in the Xn command)

Other derivatives of the +MS command:

AT+MS?	report current MS settings (e.g. 34,1,9600,33600)

AT+MS=?	list the supported values +MS:(22,122.....), (0,1), (300-33600), (300-33600)

Result codes:

OK	Syntax OK

Error	Otherwise

Related S-Reg: S31-S37

1.6 - AT+F Commands - Fax Support

+FCLASS	Data/Fax Class1/Voice mode

Sets Data/Fax Class1/Voice (0,1,8) mode

+FCLASS=<value>

[<value> - 0,1,8 (Data/Fax Class1/Voice]

Result codes:

OK	Syntax OK

Error	Otherwise

+FCLASS? Returns current setting

Related S-Regs: S32, S150

+FAE	Data/Fax Auto Answer

Select Data/Fax Auto Answer

+FAE=<value> [<value> - 0,1 (Data/Fax Class1)]

+FAE?	Returns the current setting

Related S-Regs: S151

+FTS	Stop transmission and wait

+FTS=	<value>

Terminates transmission and waits for <value>*10ms interval before responding with OK. ERROR is issued if the modem is on-hook.

+FTS?	Returns the current setting

+FRS	Receive Silence

+FRS=	<value>

Report back to DTE with OK after <value>*10ms silence interval has been detected. The command is aborted if any character is received from the DTE (The response will still be OK). ERROR is issued if modem is on-hook.

+FRS?	Returns the current setting

+FTM	Transmit Modulated Data

+FTM=<value>

Transmits data according to the defined modulation. ERROR is issued if modem is on-hook.

| Value | Modulation |
| --- | --- |
| 24.00 | V.27 ter 2400 bps |
| 48.00 | V.27 ter 4800 bps |
| 72.00 | V.29 7200 bps |
| 73.00 | V.17 7200 bps long |
| 74.00 | V.27 7200 bps short |
| 96.00 | V.29 9600 bps |
| 97.00 | V.17 9600 bps long |
| 98.00 | V.17 9600 bps short |
| 121.00 | V.17 12000 bps long |
| 122.00 | V.17 12000 bps short |
| 145.00 | V.17 14400 bps long |
| 146.00 | V.17 14400 bps short |

+FTM=? Return “24, 48, 72, 73, 74, 96, 97, 98, 121, 122, 145, 146”

+FRM	Receive Modulated Data

#### +FRM=<value>

Receives data according to the defined modulation (See Values above)

ERROR is issued if modem is on-hook.

+FRM=? Return “24, 48, 72, 73, 74, 96, 97, 98, 121, 122, 145, 146”

see +FTM

+FRH	Receive HDLC Data

+FRH=<value>

Receives data using HDLC protocol and the defined modulation. ERROR is issued if modem is on-hook.

<value> = 3 (V.21 channel 2 300 bps)

#### +FRH=? Return “3”

+FTH	Transmit HDLC Data

+FTH=<value>

Transmits data using HDLC protocol and the defined modulation. ERROR is issued if modem is on-hook.

<value> = 3 (V.21 channel 2 300 bps)

#### +FTH=? Return “3”

1.7 - AT# Commands - Voice Modem Support

Note: 	All the following commands will return OK as a result code (or ERROR if the parameters are faulty), unless stated otherwise.

#BDR	DTE Baud Rate

Sets DTE Baud Rate

#BDR=<value>

<value> - DTE Baud rate (0-48) *2400

#CID	Caller ID

Enables the Caller ID feature in any mode

#CID=<value>

#CID=0	 - Disable Caller ID

#CID=1 - Enable Caller ID (Verbose)

#CID=2 - Enable Caller ID (Numeric)

Writes the value to Sreg

Related S-Reg: S75

#RG	Receive gain level

Sets receive gain level (effects the AUDIO IN level)

#RG=<value>

<value> - 0-7fff

#TL	Transmit level

Sets transmit level (effects the AUDIO OUT level)

#TL=<value>

<value> - 0-7fff

#CLS	Data/Fax Class1/Voice mode

Same as +FCLASS=<value>

#CLS=<value>

Sets Data/Fax Class1/Voice (0,1,8) mode.

Related S-Regs: S32, S150

#VBS	ADPCM or PCM

Bits per sample (ADPCM or PCM)

#VBS=<value> [<value> - 2,4 (ADPCM), 8,16 (PCM)]

#VBS? Returns the current setting

#VBS=? Returns “2,4,8,16”

Related S-Regs: S76

#VBT	Beep tone timer

Sets Beep tone timer for generating tones and DTMF

#VBT=<value> [<value> - 0-40 (* 1/10 ms)]

#VBT? Returns the current setting

#VBT=? Returns “0-40”

Related S-Regs: S77

#VIP	Voice Parameters

Initializes Voice Parameters

Related S-Regs: S75-S89

#VIT	Inactivity timer

Sets Inactivity timer

#VIT=<value> [0-255 (* 1/10 ms)]

#VIT? Returns the current setting

#VIT=? Returns “0-255”

Related S-Regs: S19

#VLS	Voice Source selection

Select Voice Source

#VLS=<value>

0 - Telephone Line Select (Go on hook)

2 -	Speakers

3 -	Microphone

6 - Speakerphone

#VLS? Returns the current setting

#VLS=? Returns “0,2,3,6”

Result codes:

OK n=0, 6

VCON n=2, 3

ERROR Otherwise

(For 0, 6, VCON will be issued upon line connection)

Related S-Regs: S78

#VRA	Ringback Goes Away Timer (originate)

This value is used during call progress to detect a voice answer.

This is the interval between ringback ending and voice answer determined.

#VRA=<value> [0-255 (*1/10 MS)]

#VRA? Returns the current setting

#VRA=? Returns “0-255”

Related S-Regs: S79

#VRN	Ringback Never Came Timer (originate)

This value is used during call progress to detect a voice answer.

This is the interval without detection of ringback before voice answer is determined.

#VRN=<value> [0-255 (*1/10 MS)]

#VRN? Returns the current setting

#VRN=? Returns “0-255”

Related S-Regs: S80

#VRX	Voice Receive Mode

Go to Voice Receive Mode

Result codes:

CONNECT Data may be sent

ERROR VLS=0,6 and line not connected

Note: Any input from the terminal will abort Voice Receive Mode

#VSD	Silence deletion

Enables/Disables silence deletion (voice receive, ADPCM)

#VSD=<value> [0,1 - Disable/Enable]

#VSD? Returns the current setting

#VSD=? Returns “0,1”

Related S-Regs: S81

#VSP	Silence Period

Sets Silence Period (voice receive, ADPCM)

#VSP=<value> [0-255 (*1/10 ms)]

#VSP? Returns the current setting

#VSP=? Returns “0,255”

Related S-Regs: S83

#VSR	Sample Rate

Sets Sample Rate (PCM, ADPCM)

#VSR=<value> [7200, 11025, 8000]

#VSR? Returns the current setting

#VSR=? Returns “7200, 11025, 8000”

7200 is currently supported

Related S-Regs: S91

#VSS	Silence Sensitivity

Sets Silence Sensitivity (voice receive, ADPCM)

#VSS=<value> [0-3] (0-Disable, 3-allow noisy conditions

#VSS? Returns the current setting

#VSS=? Returns “0-3”

Related S-Regs: S82

#VTD	DTMF reporting capabilities

Sets DTMF reporting capabilities in Voice Transmit, Receive, and Voice Online Command Modes

#VTD=<value><value><value> [0-3F]

#VTD? Returns the current setting

#VTD=? Returns “<0-3F>,<0-3F>,<0-3F>”

Bit	Description

0	Disable/Enable DTMF detection

1	Disable/Enable V.25 1300 Hz detection

2	Disable/Enable T.30 1100 Hz detection (Fax)

3	Disable/Enable V.25/T.30 2100 Hz detection (Modem)

4	Disable/Enable Bell 2225 Hz detection

5	Disable/Enable Busy/Dial tone detection 6-7	reserved Related S-Regs: S84-S86

#VTM	timing mark

Enables timing mark placement.

#VTM=<value> [0-10 (* 1/10 ms)]

#VTM? Returns the current setting

#VTM=? Returns “0-10”

Related S-Regs: S87

#VTS	tone signal

Generates a tone signal.

#VTS= [x,y,z] | {x,z} | x, ...

[x,y,z]	x represents the first frequency (Hz)

y represents second frequency (Hz)

z represents the duration (in 100 ms units)

{x,z}	DTMF Digits with Variable Duration.

x represents the DTMF digit (0-9,A-D,*,#)

z represents the duration (in 100 ms units)

x	DTMF Digits, with duration defined by #VBT. This is represented by a value x (non-bracketed) corresponding to a DTMF digit (0-9,A-D,*,#).

#VGT	Playback Volume

Sets Playback Volume [Default 192]

#VGT=<value> [0-255 (*1/10 ms)]

#VGT? Returns the current setting

#VGT=? Returns “0-255”

Related S-Regs: S74

#VTX	Voice Receive Mode

Go to Voice Receive Mode

Result codes:

CONNECT Data may be sent

ERROR VLS=0,6 and line not connected

#SPK	Full Duplex SpeakerphoneSets Full Duplex Speakerphone parameters

#SPK=<mute>,<speaker>,<mic>

<mute>0 Microphone Mute

1 Microphone On (default)2 Room Monitor (mic on Max, Speaker off)

<speaker>

0-15 and 2-30 = dB attenuation

5 = default

16 = speaker mute

<mic>	0 - 0 dB gain

1 - 6 dB gain (Default)

2 - 9 dB gain

3 - 12 dB gain

Related S-Regs: S88-S90

S Registers Table

| S# | Function | Range | Units | W | Default | AT |
| --- | --- | --- | --- | --- | --- | --- |
| S0 | Rings to Auto-Answer | 0-255 | rings | * | 0.00 | A |
| S1 | Ring Counter | 0-255 | rings |  | 0.00 |  |
| S2 | Escape Character | 0-255 | ASCII | * | 43.00 |  |
| S3 | CR Character | 0-255 | ASCII |  | 13.00 |  |
| S4 | LF Character | 0-255 | ASCII |  | 10.00 |  |
| S5 | BS Character | 0-255 | ASCII |  | 8.00 |  |
| S6 | Wait Time for Dial Tone | 2-255 | s | * | 2.00 | D |
| S7 | Wait Time for Carrier | 1-255 | s | * | 60.00 | D |
| S8 | Pause Time for Dial (,) | 0-255 | s | * | 2.00 | D |
| S9 | Carrier Detect Response Time | 1-255 | 0.1s | * | 6.00 |  |
| S10 | Carrier Loss Disconnect Time | 1-255 | 0.1s | * | 7.00 |  |
| S11 | DTMF Tone duration | 50-255 | 0.001s | * | 70.00 | D |
| S12 | Reserved |  |  |  |  |  |
| S13 | Echo | 0-1 |  | * | 1.00 | E |
| S14 | Quiet | 0-1 |  | * | 0.00 | Q |
| S15 | Verbose | 0-1 |  | * | 1.00 | V |
| S16 | Pulse/Tone | 0-1 |  | * | 1.00 | T,P,D |
| S17 | Reserved |  |  |  |  |  |
| S18 | Test Timer | 0-255 | s | * | 0.00 | &T |
| S19 | System Inactivity Timer | 0-255 | min | * | 0.00 |  |
| S20 | Reserved |  |  |  |  |  |
| S21 | Break Length | 0-9 | 100ms | * | 9.00 | \B |
| S22 | Origin/Answer | 0-1 |  |  | 0.00 |  |
| S23 | XOFF | Character (NA) | 0-127 | ASCII | * | 19.00 |
| S24 | Flash Timer | 0-255 |  | * | 0.00 |  |
| S25 | Delay to DTR | Off (NA) | 0-255 | 0.01ms | * | 5.00 |
| S26 | RTS to CTS delay (NA) | 0-255 | 0.01ms | * | 1.00 |  |
| S27 | Auto Answer clear timeout | 0-255 | s | * | 8.00 |  |
| S28 | Pulse Set/Break Ratio | 0-4 |  | * | 0.00 | &P,P,D |
| S29 | Speaker Control | 0-3 |  | * | 1.00 | M |
| S30 | Speaker Volume | 0-3 |  | * | 1.00 | L |
| S31 | Automode Select | 0-1 |  | * | 1.00 | +MS |
| S32 | Requested Modulation (DP) |  | Mod Code | * | 34.00 | +MS |
| S33 | Requested MIN Speed |  | bps Code | * |  | +MS |
| S34 | Requested MAX Speed |  | bps Code | * |  | +MS |
| S35 | Actual Speed after CONNECT |  | bps Code |  |  |  |
| S36 | Current Data Pump Status |  | Mod Code |  |  |  |
| S37 | Actual Modulation (DP) |  | Mod Code |  |  |  |
| S38 | Actual Rx Speed |  | bps Code |  |  |  |
| S39 | Line Quality Control | 0-2 |  | * | 2.00 | %E |
| S40 | Reserved |  |  |  |  |  |
| S41 | Received Signal Level |  |  |  | 4.00 |  |
| S42 | SNR |  | dB |  |  |  |
| S43 | Result Codes control |  |  | * | 0.00 | X |
| S44 |  |  |  |  |  |  |
| S45 | Transmit Gain Level | 0-10 | -dBM | * | 3.00 |  |
| S46-S49 | Reserved |  |  |  |  |  |
| S50-S55 | Reserved for Test |  |  |  |  | &T |
| S56 | Extended Code | 0-4 |  | * | 4.00 | X |
| S57 | Reserved |  |  |  |  |  |
| S58 | Reserved |  |  |  |  |  |
| S59 | Current Setting (Zn,&Fn) | 0-5 |  | * | 3.00 | Z, &F |
| S60 | CD | 0-1 |  | * | 1.00 | &C |
| S61 | RTS | 0-1 |  | * | 1.00 | &R |
| S62 | Flow Control | 0-3 |  | * | 1.00 | &H |
| S63 | DTR | 0-3 |  | * | 0.00 | &D |
| S64 | DSR | 0-1 |  | * | 0.00 | &S |
| S65 |  | 0-1 |  | * | 1.00 | &C |
| S66 | Circuit 106 (RTS) | 0-1 |  |  | 0.00 |  |
| S67 | Circuit 107 (DSR) | 0-1 |  |  | 0.00 |  |
| S68 | Circuit 109 (CD) | 0-1 |  |  | 0.00 |  |
| S69 |  |  |  |  |  |  |
| S70 | CONNECT message format | 0-1 |  | * | 0.00 | &A |
| S71 | CONNECT message speed source | 0-1 |  | * | 0.00 | &E |
| S72-S73 | Reserved |  |  |  |  |  |
| S74 | Playback Volume | tbd |  |  |  | #VGT |
| S75 | CID Enable | 0,1 |  | * | 0.00 | #CID |
| S76 | ADPCM Bits Per Sample | 4,8,16 |  |  |  | #VBS |
| S77 | Beep Tone Timer | 0-40 | 1/10 s |  | 20.00 | #VBT |
| S78 | Line Selection Duration |  | 0,2,3,6 |  |  | #VLS |
| S79 | Ring Goes Away Timer | 0-255 | 1/10 s |  | 70.00 | #VRA |
| S80 | Ring Never Came Timer | 0-255 | 1/10 s |  | 100.00 | #VRN |
| S81 | Silence Detect Enable | 0,1 |  |  | 0.00 | #VSD |
| S82 | Silence Detect Sensitivity | 0-3 |  |  | 2.00 | #VSS |
| S83 | Silence Detect Duration | 0-255 | 1/10 s |  | 55.00 | #VSP |
| S84 | Dtmf Tone Reports Cap0 | 0-3F |  |  | 0.00 | #VTD |
| S85 | Dtmf Tone Reports Cap1 | 0-3F |  |  | 0.00 | #VTM |
| S86 | Dtmf Tone Reports Cap2 | 0-3F |  |  | 0.00 | #VTM |
| S87 | Time Mark Placement | 0-255 | 1/10 s |  | 0.00 | #VTM |
| S88 | SPK <mute> | 0-2 |  |  | 1.00 | #SPK |
| S89 | SPK <speaker> | 0-15 |  |  | 5.00 | #SPK |
| S90 | SPK <mic> | 0-3 |  |  | 1.00 | #SPK |
| S91 | Voice Sample Rate | 1,2 |  | 1.00 | 0.00 | #VSR |
| S92 | Reserved |  |  |  | 1.00 |  |
| S93-S99 | Reserved for Diagnostics |  |  |  |  |  |
| S100-S149 | Reserved for V42 |  |  |  |  | \A,\N, %C |
| S150 | FCLASS Value | 0,1,8 |  | * | 0.00 | +FCLASS |
| S151 | FAE Value | 0,1 |  | * | 0.00 | +FAE |
| S152-S160 | Reserved |  |  |  |  |  |
| S161 | Default Setting | 0-1 |  | * | 2.00 | Y |
| S162 | Country Type | 0-7 |  |  | 1.00 |  |
| S163-S169 | Reserved |  |  |  |  |  |
| S170-S174 | Debug Registers |  |  |  |  |  |
| S197-S199 | Reserved for Diagnostics |  |  | * | 2.00 |  |

Document edited by RYAN TAY on 20th May 1999